Southern Luzon State University

COLLEGE OF ENGINEERING

Computer Engineering Department

Lucban, Quezon

CPE22L: Digital Signal Processing Laboratory

Activity # 7: Z-Transform/Inverse Z-Transform

PADUADA, Dave Jhared G.

DOP: Nov 26, 2025

DOS: Dec 3, 2025

RATING

Engr. Ralph Monsour Delima

INSTRUCTOR

- Objectives

- To determine the Z-transform and Inverse Z-transform of a function.

- To identify the MATLAB codes used to generate Z-transform and Inverse Z-transform of a function.

- To understand the significance of Z-transform in digital signal processing.

- Introductory Information

THE BILATERAL Z-TRANSFORM

The z-transform is useful for the manipulation of discrete data sequences and has acquired a new significance in the formulation and analysis of discrete-time systems. It is used extensively today in the areas of applied mathematics, digital signal processing, control theory, population science, economics. These discrete models are solved with difference equations in a manner that is analogous to solving continuous models with differential equations. The role played by the z-transform in the solution of difference equations corresponds to that played by the Laplace transforms in the solution of differential equations.

The z-transform of a sequence x (n) is given by:

Xz=Zxn=n=-∞∞xnz-n

Important Properties of the Z-Transform:

1. Linearity

2. Sample shifting

3. Frequency shifting

4. Folding

5. Complex conjugation

6. Differentiation in the z-domain

7. Convolution:

Zx1n *x2n   =X1 z X2z  ;

ROC: ROCx1∩ROCx2

This last property transforms the time-domain convolution operation into a multiplication between two functions. It is a significant property in many ways. First, if X1(z) and X2(z) are two polynomials, then their product can be implemented using the conv function in MATLAB.

- Data & Procedures

| conv_m function [y, ny] = conv_m(x, nx, h, nh) % Modified convolution routine from Source [45-50] nyb = nx(1) + nh(1); nye = nx(length(x)) + nh(length(h)); ny = [nyb:nye]; y = conv(x, h); end |  |
| --- | --- |
| Exercises 7.1. Determine the results of the following polynomial operations using MATLAB 𝑎) X1(z) = (1-2z-1+3z-2-4z-3) (4 +3z-1-2z-2+z-3) 𝑏) X2(z) = (z2 - 2z +3+2z-1) (z3-z-3) disp('--- Result for Exercise 7.1 (a) ---'); % Coefficients for first polynomial: 1, -2, 3, -4 x1_a = [1, -2, 3, -4]; % Indices (powers of z^-1): 0, 1, 2, 3 n1_a = [0:3]; % Coefficients for second polynomial: 4, 3, -2, 1 x2_a = [4, 3, -2, 1]; % Indices (powers of z^-1): 0, 1, 2, 3 n2_a = [0:3]; % Perform convolution using conv_m [y_a, ny_a] = conv_m(x1_a, n1_a, x2_a, n2_a); disp('Coefficients (y):'); disp(y_a); disp('Indices/Powers (n):'); disp(ny_a); disp('--- Result for Exercise 7.1 (b) ---'); % Poly 1: 1z^2 - 2z^1 + 3z^0 + 2z^-1 % Coefficients: [1, -2, 3, 2] x1_b = [1, -2, 3, 2]; % Indices start at -2 (for z^2) and end at 1 (for z^-1) n1_b = [-2:1]; % Poly 2: 1z^3 + 0z^2 + 0z^1 + 0z^0 + 0z^-1 + 0z^-2 - 1z^-3 % We must include zeros for missing terms between z^3 and z^-3 x2_b = [1, 0, 0, 0, 0, 0, -1]; % Indices start at -3 (for z^3) and end at 3 (for z^-3) n2_b = [-3:3]; % Perform convolution using conv_m [y_b, ny_b] = conv_m(x1_b, n1_b, x2_b, n2_b); disp('Coefficients (y):'); disp(y_b); disp('Indices/Powers (n):'); disp(ny_b); | Exercise 7.2 Determine the following inverse z-transforms using the partial fraction expansion method. 𝑎) x1(z) = 1-z-1-4z-2+4z-3 1-114z-1+138z-2-14z-3 𝑏) x2z=z3-3z2+4z+1z3-4z2+z-0.16 %% Exercise 7.2 (a) % Equation: (1 - z^-1 - 4z^-2 + 4z^-3) / (1 - 11/4z^-1 + 13/8z^-2 - 1/4z^-3) disp('--- Results for Exercise 7.2 (a) ---') % Coefficients of the Numerator (b) b1 = [1, -1, -4, 4]; % Coefficients of the Denominator (a) % Note: 11/4 = 2.75, 13/8 = 1.625, 1/4 = 0.25 a1 = [1, -11/4, 13/8, -1/4]; % Calculate Residues (R), Poles (p), and Direct Terms (C) [R1, p1, C1] = residuez(b1, a1); disp('Residues (R):'); disp(R1); disp('Poles (p):'); disp(p1); disp('Direct Terms (C):'); disp(C1); %% Exercise 7.2 (b) % Equation: (z^3 - 3z^2 + 4z + 1) / (z^3 - 4z^2 + z - 0.16) % Converted to z^-1 format by dividing top and bottom by z^3 disp('--- Results for Exercise 7.2 (b) ---') % New Numerator: 1 - 3z^-1 + 4z^-2 + 1z^-3 b2 = [1, -3, 4, 1]; % New Denominator: 1 - 4z^-1 + 1z^-2 - 0.16z^-3 a2 = [1, -4, 1, -0.16]; % Calculate Residues (R), Poles (p), and Direct Terms (C) [R2, p2, C2] = residuez(b2, a2); disp('Residues (R):'); disp(R2); disp('Poles (p):'); disp(p2); disp('Direct Terms (C):'); disp(C2); |
| Exercise 7.3 Plot the zero-pole diagram for: a. Xz =11-0.9z-1 1+0.9z-1 b. Xz =z3z2-4z+1 %% Exercise 7.3 (a) % Equation: X(z) = 1 / [ (1 - 0.9z^-1) * (1 + 0.9z^-1) ] % Expanding the denominator: (1 - 0.9z^-1)(1 + 0.9z^-1) = 1 - 0.81z^-2 % This means the denominator polynomial is: 1 + 0z^-1 - 0.81z^-2 disp('Generating Zero-Pole Plot for Exercise 7.3 (a)...') % Numerator coefficients (just 1) num_a = [1]; % Denominator coefficients (1, 0, -0.81) den_a = [1, 0, -0.81]; % Create a new figure window figure(1); zplane(num_a, den_a); title('Zero-Pole Diagram for Exercise 7.3 (a)'); grid on; %% Exercise 7.3 (b) % Equation: X(z) = z^3 / (z^2 - 4z + 1) % To use zplane with coefficients, we usually prefer negative powers of z. % Divide numerator and denominator by z^2 (highest power of denominator). % Numerator: z % Denominator: 1 - 4z^-1 + z^-2 disp('Generating Zero-Pole Plot for Exercise 7.3 (b)...') % Numerator coefficients: z corresponds to [1, 0] (representing 1*z + 0) num_b = [1, 0]; % Denominator coefficients: 1 - 4z^-1 + 1z^-2 den_b = [1, -4, 1]; % Create a new figure window figure(2); zplane(num_b, den_b); title('Zero-Pole Diagram for Exercise 7.3 (b)'); grid on; |  |

- Data and Results

| Exercises 7.1 (a) | Program7_1a --- Result for Exercise 7.1 (a) --- Coefficients (y): 4 -5 4 -2 -20 11 -4 Indices/Powers (n): 0 1 2 3 4 5 6 X1z=4-5z-1+4z-2-2z-3-20z-4+11z-5-4z-6 The input consisted of two polynomials that used only negative powers of z(such as z-1,z-2), which in digital signal processing correspond to causal signals beginning at time n=0. When these polynomials were convolved, the standard convolution operation simply combined their coefficients, and because both inputs effectively began at index 0, the resulting polynomial also began at index 0. The output therefore produced a longer sequence that extended further into the negative powers, reaching as far as z-6. |
| --- | --- |
| Exercise 7.1 (b) | >> Program7_2b --- Result for Exercise 7.1 (b) --- Coefficients (y): 1 -2 3 2 0 0 -1 2 -3 -2 Indices/Powers (n): -5 -4 -3 -2 -1 0 1 2 3 4 X2z=z5-2z4+3z3+2z2-z-1+2z-2-3z-3-2z-4 In this case, the input polynomials included both positive and negative powers of z, representing a non-causal signal that exists for negative time indices (n<0). The standard MATLAB conv function does not account for the actual starting index of the sequences; it assumes all sequences start at index 1. In contrast, the custom function conv_m correctly computed the true starting index of the convolution by adding together the starting indices of the two input sequences—for example, (-2)+(-3)=-5—which allowed it to align the output properly in time. |
| Exercise 7.2 (a) | >> Program7_2a Program7_2a --- Results for Exercise 7.2 (a) --- Residues (R): -0.0000 -10.0000 27.0000 Poles (p): 2.0000 0.5000 0.2500 Direct Terms (C): -16 X1z= -16-101-0.5z6-1+271-0.25z-1 The term for the pole at p = 2 is omitted because its residue is 0. Exercise 7.2 (a) The MATLAB residuez function performed a Partial Fraction Expansion (PFE) on the rational function. The calculation resulted in a direct term (C) of -16, which arises because the numerator and denominator have equivalent highest powers of z^−1 (specifically the z^−3 terms), requiring polynomial division. You will notice that the pole at z=2 has a residue of effectively zero (0.0000). This indicates a pole-zero cancellation, meaning the numerator also contained a zero at z=2 that canceled out the denominator's pole. Consequently, this term disappears from the final equation, leaving only the terms for the poles at z=0.5 and z=0.25. |
| Exercise 7.2 (b) | >> Program7_2b --- Results for Exercise 7.2 (b) --- Residues (R): 0.5383 + 0.0000i 3.3559 + 5.7659i 3.3559 - 5.7659i Poles (p): 3.7443 + 0.0000i 0.1278 + 0.1625i 0.1278 - 0.1625i Direct Terms (C): -6.2500 X2z=z5-2z4+3z3+2z2-z-1+2z-2-3z-3-2z-4 For this function, the expansion resulted in complex numbers. The polynomial roots (poles) included a real pole at approximately 3.74 and a pair of complex conjugate poles at 0.1278±0.1625i. In Z-transform theory, complex poles always appear in conjugate pairs for polynomials with real coefficients, and they typically represent oscillatory behavior in the time domain. As with the previous exercise, a direct term (C=−6.25) is present because the highest degree of the numerator matches the denominator (both were converted to have terms up to z^−3), necessitating a division of the leading coefficients (1/−0.16) before separating the fractions. |
| Exercise 7.3 (a) | The Pole–Zero Diagram shows two poles located on the horizontal axis at –0.9 and 0.9, along with a double zero at the origin indicated by the circle labeled “2.” The poles match the expected roots of the denominator (1-0.9z-1)(1+0.9z-1), giving z=0.9and z=-0.9. Because both poles lie inside the unit circle, the system is stable. |
| Exercise 7.3 (b) | The diagram for this function displays one pole near 0.27 inside the unit circle and another around 3.73 outside the circle. These correspond to the roots of the polynomial z2-4z+1. Since one pole lies outside the unit circle, the system is unstable; in digital signal processing, even a single pole outside the unit circle causes the output to grow without bound. |

- Conclusion

Based on the objectives and results of Activity 7, we can draw the following conclusions:

Computational Efficiency with MATLAB: The Z-transform significantly simplifies the analysis of discrete-time systems. As demonstrated in Exercise 7.1, complex convolution operations in the time domain are reduced to simple polynomial multiplication in the Z-domain. MATLAB functions like conv (and the custom conv_m) allow us to easily calculate these system responses without manually performing tedious summations.

Inverse Transformation via Partial Fractions: Exercise 7.2 showed that we can reverse the process using the Partial Fraction Expansion method. The MATLAB function residuez is a powerful tool that automates the decomposition of complex rational functions into simpler terms, allowing us to easily determine the time-domain sequence x(n) from a frequency-domain function X(z).

System Stability Analysis: Exercise 7.3 highlighted the importance of the Pole-Zero plot. The location of the poles (the roots of the denominator A(z)) is the defining factor for system stability. By using zplane, we can visually inspect whether a system is stable. If all poles lie within the unit circle, the system is stable; if any pole lies outside, the system is unstable.

This activity successfully demonstrated that the Z-transform is the discrete-time equivalent of the Laplace transform, serving as an essential tool in Digital Signal Processing for analyzing system behavior, stability, and response. MATLAB serves as an effective bridge to visualize these abstract mathematical concepts.