CPE22 – Embedded Systems

Devices and Communication Buses for Devices Network

I/O – Input/Output

SI – Serial Interface

SPI – Serial Peripheral Interface

PPI – Parallel Peripheral Interface

GPIO – General Purpose Input Output

IBM – International Business Machines Corporations
UART – Universal Asynchronous Receiver-Transmitter

USART – Universal Synchronous-Asynchronous Receiver-Transmitter

TX – Transmit

RX – Receive

Serial Communication Protocols:

RS-### - Recommended Standard

I2C – Inter-Integrated Circuit

CAN –Controller Area Network

USB – Universal Serial Bus

1-Wire

| IO Port types - Serial and Parallel IO ports |  |
| --- | --- |
| IO Port | - A device that is used to receive bytes from external peripheral(s), device(s), processor(s), or controller(s) for later reading using instructions executed on the processor. - A device that is used to send bytes to an external peripheral, device, or processor using instructions executed on the processor. |
| Port | - A port is an interface (connection point) between the processor (CPU) and an external device (like a keyboard, display, sensor, or other peripheral). Uses the addresses: - It connects to the processor through an address decoder and the system buses (data bus, address bus, control bus). Uses Port-Register Addresses: - Program’s the port’s functions or modes, - Read the port’s status, and - Write or read data bytes to or from the connected device. |
| Port buffer | A port buffer is used to temporarily hold the data being transferred. |
| Microcontroller/Microprocessor Interfaces Examples |  |
| SI (Serial Interface) in 8051 | A serial communication port in 8051 microcontroller for sending/receiving data one bit at a time. |
| SPI (Serial Peripheral Interface) in 68HC11 | A high-speed serial communication protocol that allows the 68HC11 microcontroller to talk to peripherals (like sensors, displays, or memory chips). |
| PPI (Parallel Peripheral Interface) 8255 | An Intel 8255 chip that provides a parallel port transferring multiple bits of data at the same time to/from external devices. |
| Microcontroller Ports |  |
| Ports P0, P1, P2, P3 (8051) | Built-in general purpose I/O (GPIO) ports on the 8051 microcontroller for digital input/output. |
| Ports PA, PB, PC, PD (68HC11) | Similar general purpose I/O ports in the 68HC11 microcontroller. |
| COM1 and COM2 (IBM PC) | Serial communication ports (RS-232) on older IBM PCs used to connect devices such as modems, mice, or printers. |
| IO Port Types |  |
| Serial Port | Transfers data one bit at a time over a single communication line. It is slower than parallel for short distances but allows long-distance communication with fewer wires (e.g., UART, SPI, modem connections). Types of Serial Ports: Synchronous Serial Input; Data bits are received in sync with a clock signal.. Synchronous Serial Output; Data bits are sent in sync with a clock signal.. Asynchronous Serial UART Input; Data is received without a shared clock; timing is managed by start/stop bits.. Asynchronous Serial UART Output; Data is sent without a clock, also using start/stop bits.. Both as input and output (Full - DUPLEX); Can send and receive simultaneously on separate lines.. |
| Parallel Port | Transfers multiple bits at the same time using several data lines. It is faster for short-distance communication but requires more wires and is more prone to signal interference (e.g., printer port, 8255 PPI) Types of Parallel Ports Parallel Port One-Bit Input; Reads a single digital signal (HIGH or LOW). Parallel Port One-Bit Output; Sends a single digital signal (HIGH or LOW). Parallel Port Multi-Bit Input; Reads a group of bits (like 4) at the same time. Parallel Port Multi-Bit Output; Sends a group of bits simultaneously. |
| Parallel Input/Output |  |
| Input/Output | Is the information exchange between CPU and (external) connected devices. - Interfacing and programming I/O devices: Special instruction for hardware. Different from our previous programs. - Electrical characteristics different from CPUs. Analog devices, power, current drive - I/O devices operate asynchronously from the CPU (and the program being run). Transfers data: processor and I/O device synchronize or “handshake” to exchange information Independent I/O components associated with each connected device. |
| I/O Components | - Interfaces that “electrically” connect external device to computer’s internal bus. - Bus connection allows CPU to read and/or write device |
| I/O Addresses | - When connected to a computer system, each port is assigned an I/O address - Device (port) identified by it’s I/O addresses - CPU read/write from/to I/O address to receive/send data from/to device. 2 kinds of I/O Addresses: - Isolated I/O or Port-mapped I/o - Memory-Mapped I/o |
| I/O Addressing Schemes |  |
| Isolated I/O | - Separate address space for I/O devices (different from memory). - Uses special instructions for I/O: IN and OUT. - CPU has dedicated control signals for I/O read/write. - I/O ports have their own unique address (e.g., 0–FFFFh). - Example: 80x86 family uses this scheme. |
| Memory-Mapped I/O | - I/O devices share same address space as memory. - CPU uses regular data transfer instructions (like MOV, ADD, etc.) for both memory and I/O. - No separate I/O instructions — memory and I/O treated the same way. - Simpler addressing but consumes part of the memory address space. |
| Switch De-bouncing | - A method to remove noise or oscillations caused by mechanical contact bounce when a switch changes position. Prevents multiple readings for a single press. - Program reading switch port, “value” of switch output will oscillate (open/closed) until bouncing stops - Program must filter out oscillations so that program only “sees” one switch state change per position change. - The overall process of correcting for the rapid on-and-off contact of a physical switch as it closes, which can cause multiple false triggers in a fast digital circuit. The goal is to get a single, clean signal for each press or release of the switch. |
| Simple De-bouncing | - Program waits a fixed “enough time” after detecting a change before reading the stable value. Implemented using a delay loop. - A basic method that uses a fixed delay. |
| Adaptive De-bouncing | - The program polls continuously; every time a change is detected, a loop counter resets. When the counter reaches 0 (no changes for a while), the input is considered stable. - An advanced method that dynamically adjusts the delay time. |
| Synchronous Serial Communication | A data transmission method where data bits are synchronized with a clock signal (SCLK). Each bit in every byte is sent or received in sync with clock pulses. |
| Clock Signal | Ensures the sender and receiver are synchronized. The clock may be sent on a separate line or combined/encoded with the data line. |
| Synchronization | Bits/bytes are received at constant time intervals based on the clock (T = clock period). Each byte takes 8T to receive. Data rate = 1/T bps. |
| Data Transfer Rate (Synchronous) | If clock period = T, data transfer rate = 1/T bits per second (bps). Each byte takes 8T time to send or receive. |
| Port Register (for Serial Input) | An 8-bit register in the peripheral that temporarily stores received serial bits before being read by the microprocessor. |
| Synchronous Serial Input | Each bit received is synchronized with the clock. The port contains a shift register that collects 8 bits and stores them in the port register for CPU reading. |
| Synchronous Serial Output | Each bit is transmitted in sync with the clock signal. The microprocessor writes a byte into the port’s shift register, which shifts data out bit-by-bit with each clock pulse. |
| Synchronous Serial Input/Output (Combined) | Both input and output operations are synchronized with the same master clock; data rate = 1/T bps. I/O ports may share the same line if clock pulses are modulated or encoded. |
| MOSI (Master Out, Slave In) | Data line where the master sends data to the slave while synchronizing with the master’s clock (SCLK). Used as the SPI pin for data transmission from master to slave. |
| MISO (Master In, Slave Out) | Data line where the slave sends data to the master, synchronized by the master’s clock. Used as the SPI pin for data transmission from slave to master. |
| Asynchronous Serial Communication | A data transmission method where no clock signal is shared. Each device uses its own timing (same baud rate). Data bits arrive at fixed bit intervals, but bytes are not synchronized. Asynchronous Serial Input Examples - Keyboard, Mouse, Modem, Keypad, Serial COM Ports Asynchronous Serial Output Examples - Printer, Modem Output, Serial Line Output |
| UART (Universal Asynchronous Receiver-Transmitter) | A hardware protocol that implements asynchronous serial communication. Handles conversion between parallel and serial data formats. |
| UART Input (RxD – Receive Data) | Input line on a UART COM port; receives serial data bits without an external clock. Example: PC COM1/COM2 RxD line. |
| UART Output (TxD – Transmit Data) | Output line that sends serial data bits asynchronously. Each bit transmitted at constant bit time T, but bytes may be separated by variable intervals. |
| Baud Rate | The number of symbol changes or signal events per second transmitted over a data communication channel The number of bits transmitted per second (bps). If bit period = T, baud rate = 1/T. |
| UART Frame Format | Consists of: • Start bit (1→0 transition) – marks start of data • Data bits (7–9 bits typical) • Optional parity bit – for error checking • Stop bit(s) (1 or 2 bits, logic ‘1’) – marks end of frame |
| UART Receiver Operation | Detects start bit (line goes LOW), samples data bits at mid-bit times, stores them in a shift register, and finally transfers the byte to a port register for the CPU. |
| Half Duplex | Communication where data transmission can occur only one direction at a time on a bi-directional line. Example: Telephone communication — only one person talks at a time. |
| Full Duplex | Communication where data transmission occurs in both directions simultaneously. Example: Modem–Computer communication via TxD and RxD lines, or 8051 microcontroller SI modes 1–3. |
| Parallel Port (Single-Bit Input) | Used when a single signal or event needs to be detected. Examples: Completion of a wheel revolution, reaching preset boiler pressure, detecting object weight limit, sensing a magnetic piece for robot arm reach, or detecting liquid level. |
| Parallel Port (Single-Bit Output) | Used to control or send one-bit signals to external devices. Examples: PWM output for DAC (controls liquid level, temperature, pressure, or motor speed), generating pulses or control signals to external circuits. |
| Parallel Port (Multi-Bit Input) | Reads multiple data bits simultaneously (e.g., 8-bit or 16-bit). Examples: ADC input from sensors (temperature, pressure, motor speed), encoder inputs for angular position or linear displacement. |
| Parallel Port (Multi-Bit Output) | Sends multiple bits at once to external devices or displays. Examples: LCD controller in cellphones (for text/images), print controller outputs, stepper motor coil driver bits. |

Serial Communication Methods: Synchronous and Asynchronous

Synchronous: I^2C & SPI

Asynchronous: UART & CAN

| Serial Communication Methods |  |
| --- | --- |
| Serial Communications | - describes a general way in which two devices can send or receive data from one another. - require fewer I/O (input-output) lines |
| Parallel Communications | - a chunk of data (8,16 or 32 bit) is sent at a time - each bit of data requires a separate physical I/O line |
| Serial | data bits are transmitted sequentially, one bit at a time. - Using only one wire pair - In parallel communication, multiple bits are transmitted simultaneously using multiple wires. Physical Transmission of Serial Data - USB - Firewire - Ethernet - RS-232 - RS-422 - RS-485 Others: BNC’s, DB9, HDI5, Mini Centronics 60, Mini D50, JW3, DB15, DB25, Keyed 50 Pin Header, 5W5, Mini DIN 8, IEEE 488, Centronics 50, 21WA4, V35, Mini D68, Type A, Type B, Mini-A, Micro-AB, & Micro B. |
| Data | In the form of binary pulses |
| Serial Communications |  |
| Simplex Method | One-way communication technique. |
| Half-Duplex Mode | - Both sender and receiver are active but not at a time - Communication mode where data transmission occurs one direction at a time (e.g., 1-Wire). - Protocols that send and receive alternately (e.g., RS485, 1-Wire). |
| Full Duplex Mode | - Both sender and receiver can transmit and receive at the same time. - Communication mode allowing simultaneous transmission and reception (e.g., RS232). - Protocols allowing simultaneous send and receive (e.g., RS232, USB). |
| Serial Communication Methods – Synchronous and Asynchronous |  |
| Synchronous Method | A serial communication method where data transfer is synchronized with a shared clock signal (SCK or SCLK). Requires additional wire for clock. Example: SPI, I²C, USB. |
| Asynchronous Method | Serial communication without a shared clock signal. Devices communicate using predefined baud rates. Example: UART, RS232, CAN. |
| Clock Signal (SCK/SCLK) | The signal that synchronizes data transmission timing between sender and receiver in synchronous systems. |
| SS (Slave Select) | SPI pin used by the master to select which slave device to communicate with. |
| USART (Universal Synchronous-Asynchronous Receiver-Transmitter) | A communication module capable of both synchronous and asynchronous transmission. |
| UART (Universal Asynchronous Receiver-Transmitter) | A hardware communication protocol that transmits data serially without a clock, using start and stop bits for synchronization. |
| USB (Universal Serial Bus) | - A high-speed serial protocol that supports both synchronous communication and data transfer up to 4.8 Gbps (USB 3.0). Commonly used for connecting peripherals. - supports a wide range of speeds, from 1.5 Mbps (USB 1.0) up to 4.8 Gbps (USB 3.0), making it much faster than older interfaces. - In microcontroller-based systems, USB OTG is commonly used for dumping hex files (program code) directly to the microcontroller for programming. |
| RS-232 | A standard serial communication protocol using TX and RX lines. Supports asynchronous mode (used in PCs) and synchronous (not commonly used). |
| RS-422 | An improved version of RS232 using differential twisted-pair cables for longer distances (up to 4000 ft) and higher speed (up to 10 Mbps). |
| RS-485 | A multi-drop serial protocol supporting up to 32 transmitters and 32 receivers, ideal for industrial networks. Only one transmitter active at a time. |
| CAN (Controller Area Network) | A message-oriented asynchronous serial protocol used in automotive and industrial systems for reliable multi-node communication without a clock signal. |
| I²C (Inter-Integrated Circuit) | A two-wire synchronous protocol (SDA for data, SCL for clock) that supports multiple devices using 7- or 10-bit addressing. Speed: up to 400 kbps. |
| SPI (Serial Peripheral Interface) | - SPI protocol sends and receives data in a continuous stream without interruption and is recommended when high-speed communication is required. - A synchronous serial protocol using 4 wires (MOSI, MISO, SCK, SS). Fast communication (up to 10 Mbps), suitable for high-speed data transfer. - Unlimited slaves can be connected theoretically, but in practice the number depends on the load capacitance of the bus. |
| Microwire | A 3-wire serial protocol derived from SPI. Faster than I²C, supports speeds up to 3 Mbps. |
| 1-Wire Protocol | Asynchronous communication using a single data line plus ground. Provides half-duplex communication and uses internal clocking. |
| Baud Rate | The number of bits transmitted per second (bps). Both devices must have the same baud rate. Common values: 9600, 57600, 115200. |
| Framing | Defines the number of data bits (usually 8) sent in one frame. Determines how the receiver interprets each group of bits. |
| Endianness | Refers to the byte order used when transmitting or storing multi-byte data. Can be big-endian (MSB first) or little-endian (LSB first). |
| Synchronization (Bits) | - Process of adding a start bit and one or two stop bits to identify the beginning and end of data during asynchronous transmission. - Bits help the receiver to identify the start and end of the data transfer - Transmitter appends synchronization bits (1 Start bit and 1 or 2 Stop bit) to the original data frame. |
| Parity Bit (Error Control) | - Used to detect errors in transmission. Even parity = even number of 1’s, odd parity = odd number of 1’s. - Data corruption may happen due to external noise at the receiver end. The only solution to get the stable output is to check the Parity. |
| Differential Signaling | Method using two wires carrying inverted signals to reduce noise (used in RS422, RS485). |
| Start Bit | In asynchronous transmission, a bit (always 0) signaling the start of a data frame. |
| Stop Bit | Bit (always 1) indicating the end of a data frame in asynchronous communication. |
| Data Frame | A complete set of bits including data bits, start bit, stop bit, and optional parity bit transmitted as a unit. |
| Master Device | The controller that initiates and controls data transfer in synchronous systems (SPI, I²C). |
| Slave Device | A peripheral device that responds to the master’s request in synchronous communication. |
| Bus | Shared communication line connecting multiple devices for data transfer. |
| Multiplexing | Technique of sending multiple signals over a single line or channel. CAN protocol uses multiplexing. |

Periodic Interrupt, Waveform Generation, & Time Measurement

| Interrupt | When a peripheral device needs help during program execution, it sends an interrupt signal to the microcontroller. The microcontroller then pauses the main program and runs a special program called the Interrupt Service Routine (ISR) to handle the request. Once the ISR is finished, the microcontroller goes back to continue running the main program. Classification of Interrupts External & Internal Interrupts; External interrupts are those initiated by peripheral devices through the external pins of the microcontroller Internal interrupts are those activated by the internal peripherals of the microcontroller like timers, serial controller etc.). Maskable & Non-Maskable Interrupts; The category of interrupts which can be disabled by the processor using program is called maskable interrupts. Non-maskable interrupts are those category by which the programmer cannot disable it using program.. Vectored & Non-Vectored Interrupt; In vectored interrupts the starting address is predefined. In non-vectored interrupts, the starting address is provided by the peripheral as follows.. |
| --- | --- |
| IE Register (Interrupt Enable) | 8-bit register used for enabling or disabling the interrupts |
| IP Register (Interrupt Priority) | 8-bit register used for setting the priority of the interrupts |
| Timers/Counters | Are generally used for: - Time Reference - Creating Delay - Waveform properties measurement - Periodic Interrupt Generation - Waveform Generation Timer is always incremented by the microcontroller. The time taken to count one digit up is based on master clock frequency. Timer Modes: Timer modes define how a microcontroller’s timer/counter operates to measure time intervals or count external events. Each mode determines how the timer registers (THx and TLx) function and how the timer overflows or generates interrupts Timer Mode-0; 13-bit timer using 5 bits of TLx and 8 bits of THx; used for measuring pulse width.. Timer Mode-1; 16-bit timer for longer count ranges.. Timer Mode-2; 8-bit auto-reload timer where TLx counts and THx holds the reload value. Timer mode-3; Splits Timer 0 into two 8-bit timers, while Timer 1 stops counting.. |
| Watchdog Timer | - is a piece of hardware in micro-controller. It is used to generate system reset if system gets stuck somewhere. - if system goes into endless loop of execution watchdog timer will reset the system to come out of endless loop. - Can be thought of as having the inverse functionality than that of a regular timer. We configure a watchdog timer with a real-time value, just as with a regular timer - Watchdog is basically a “COUNTER”, which starts from counting zero and reaches to certain value then watchdog hardware will generates a watchdog reset. To avoid system reset, software needs to kick the watchdog. |
| Real-Time Clock (RTC) | - is a small electronic device used in various systems, especially in embedded systems, to keep track of the current time. - It functions like a regular clock that we see in our homes, but it works inside computers, machines, and many other electronic devices. - It uses a special piece of hardware called a quartz crystal to create tiny, regular electrical signals. These signals act like a ticking sound inside the RTC, allowing it to keep track of time. - Battery: CR2032 |
| Waveform Generation |  |
| Digital to Analog Converter (DAC) | - is a device with a function opposite to that of an analog to digital converter, i.e. it converts a digital word to a corresponding analog voltage. - is an electronic device that transforms digital audio signals (like those from a CD, computer, or phone) into analog audio signals, which can then be understood and amplified by headphones and speakers to produce sound. 3 Integer Formats - 8-bit – LS byte of data hold register - 12-bit right aligned – the 12 LS bits of the data hold register - 12-bit left aligned – the 12 MS bits of the data hold register - Has 2 output channels, can be done independently or simultaneously - TIM6 & TIM7 are basic timers and are intended for DAC triggering |
| STM32 | - is better for advanced, high-performance applications requiring more processing power, peripherals, and scalability. High Performance; Mainstream; Ultra-low Power; Wireless. STM32H5; STM32C0; STM32L0; STM32WB0. STM32H7; STM32F0; STM32L4; STM32BWA. STM32F2; STM32F1; STM32L4+; STM32WB. STM32F4; STM32F3; STM32U0; STM32WL. STM32F7; STM32G0; STM32U3. STM32N6; STM32G4; STM32U5. - has at least one Direct Memory Access (DMA) module with multiple channels (streams) - “DMA under run error” flag |
| STM32 DAC | - The STM32 DAC module is a 12-bit word converter, with up to three output channels to support audio functions. - can also be used for many other purposes, such as analog waveform generation and control engineering. |
| Sine waveform | - is also called a monotone signal, it is known for pure tone. The sine tones are traditionally used as stimuli to assets the response of the audio system. |
| White Noise Generator | *a circuit or program that creates a random signal with a flat spectral density, meaning it contains all audio frequencies at equal intensity - A device or circuit that produces a random signal with equal intensity across all frequencies, resulting in a flat spectrum. In embedded systems, it often uses a pseudo-random code generator that generates sequences up to 2n-1 numbers, producing a uniform (not Gaussian) noise distribution. |
| Noise with offset | - Noise waveform + offset = new signal - Can combine with pattern waveform - Flexible for different uses Typical Applications ADC Testing & Calibration - Oversampling & Calibration - Improve Accuracy Audio/Music Synthesis - Cymbals, Percussive Sounds - Create other noise-based signals Electronic & Engineering - Filter & Amplifier Testing - Frequency response measurement |
| Triangular wave generator | - Includes a triangular waveform generator - Generator supports programmable offset, amplitude, and frequency - Amplitude is set using MAMPx bits in the DAC_CR register Typical Applications - Triangular wave generators are common in sound synthesis - Produce a softer timber compared to square waves Example of use: - Analog synthesizers (Moog, Roland) - Digital music synthesizers & software plugins - Electronic organs - Sound effects in retro video games - Signal modulation in audio processing |
| Buffered Output | DAC channels have embedded output buffers (enable/disable depending on application) Without buffer: - Output voltage drops when driving a load due to high DAC output impedance. |
| Class Rules | - Be respectful - Be engaged - Be prepared - Be safe - Be responsible |
| Using the DAC to implement an audio player | This shows a system that can play .WAV files from an SD Card using a STM32 microcontroller. Audio Wave file specifications - Format: PCM (uncompressed wave data format) - Sample Rate: 8lHz, 11.025 kHz, 22.05 kHz, or 44.1 kHz - Resolution: 8-bit (Values from 0-255) - Channels: Mono (1 Channel) |
| .WAV file format | - is a subset of the Resource Interchange File Format (RIFF) specification used for the storage of multimedia files. - A .WAV file is often just a RIFF file with a single "WAVE" chunk consisting of two sub-chunks: - fmt chunk – describes audio format (PCM, channels, sample rate, bits per sample) - data chunk – holds the actual audio samples |
| Disk Operating File System (DOSFS) | - is a lightweight file system used in embedded systems to manage files stored on devices like microSD cards. |

Data Acquisition, Control, Sensors, & Actuators

DMM – Digital Multimeter

| Personal Computer (PC) | - Used to program test equipment and manipulate or store data. - Includes any computer running any OS and software that supports the desired result. - also used for supporting functions, such as real-time graphing or report generation. - may not necessarily be in constant control or even connected to the data acquisition equipment at all times. |  |
| --- | --- | --- |
| Test Equipment | Can consist of: - Data acquisition plug-in boards for PCs, - External board chassis - Or discrete instruments External chassis or discrete instruments are typically connected to a PC using either: - Standard communication ports - Or a proprietary interface board in the PC Can perform one or more measurement and control processes. These are: - Analog I/O - Digital I/O - Or other specialized functions |  |
| Data Acquisition and Control | Broadly defers to a variety of hardware and software solutions capable of making measurements and controlling external processes. |  |
| Computer | Means an IBM-compatible PC running Microsoft Windows 95 or later, unless otherwise noted. |  |
| Data Acquisition & Control Hardware | Available in various forms, with varying levels of: - Functionality - Channel count - Speed - Resolution - Accuracy - Cost Form/Categories Various Categories (based on board cross-section) include: - Plug-in Data Acquisition Boards - External Data Acquisition Systems - Real-time Data Acquisition and Control - Discrete (Bench/Rank) Systems - Hybrid Data Acquisition System |  |
| Plug-In Data Acquisition Boards | - Designed for mounting in board slots on a computer motherboard. - Buses it is mostly designed for: - Current PCI (Peripheral Component Interconnect) buses - Or earlier ISA (Industry Standard Architecture) buses - It is also designed for these buses: - EISA - IBM Micro Channel - And various Apple buses But these are no longer considered mainstream products. Category Characteristic: - Variety of Test Functions - High Channel Counts - High Speed - Adequate sensitivity to measure moderately low signal levels - Low cost |  |
| Features of Plug-in Data Acquisition Boards |  |  |
| 1 | Least expensive method of computerized measurement and control |  |
| 2 | High speed available (100kHz to 1Ghz and higher) |  |
| 3 | Available in multi-function versions that combine A/D, D/A digital I/O, counting, timing, and specialized functions. |  |
| 4 | Good for tasks involving low-to moderate channel counts |  |
| 5 | Performance adequate to excellent for most tasks, but electrical noise inside the PC can limit ability to perform sensitive measurements. |  |
| 6 | Input voltage range is limited to approximately ± 10V |  |
| 7 | Use of PC expansion slots and internal resources can limit expansion potential and consume PC resources |  |
| 8 | Making or changing connections to board’s I/O terminals can be inconvenient |  |
| Key Features of Modern External Systems - High Sensitivity – can detect very low-level voltage signals (<1mV or lower) - Support – Support real time process control needs - Suitable for - Wide variety of sensors - High channel counts - Stand-alone operation System Architecture - Architectures include : VME, VXI, MXI, CompactPCI, PXI - Requires computer for operation and data storage - Features: - Robust standardized boards - Plug-in instrument modules with full test and measurement functions Advanced Capabilities - Some designed include microprocessor modules with - Keyboard, monitor, mouse support - Standard communication port - Capabilities – often capable of running Microsoft Windows and PC applications - Conventional PCs mainly used for - Program development - Off-loading data for analysis/manipulation |  |  |
| Features of External Data Acquisition Chasis |  |  |
| 1 | Multiple board slots permit mixing-and-matching boards to support specialized acquisition and control tasks and higher channel counts. |  |
| 2 | Chassis offers an electrically quieter environment than a PC, allowing for more sensitive measurements. |  |
| 3 | Use of standard interfaces (IEEE-488, RS-232, USB, FireWire, Ethernet) can facilitate daisy chaining, networking, long distance acquisition, and use with non-PC computers. |  |
| 4 | Dedicated processor and memory can support critical “real-time” control applications or stand-alone acquisition independent of a PC. |  |
| 5 | Standardized modular architectures are mechanically robust, easy to configure, and provide for a variety of measurement and control functions. |  |
| 6 | Required chassis, modules, and accessories are cost-effective for high channel counts. |  |
| 7 | Some architectures have minimal vendor support, limiting the sources of equipment and accessories available. |  |
| Real-time Data Acquisition & Control | - Critical real-time control is an important issue in data acquisition and control systems. - Applications that demand real-time control are typically better suited to external systems than to systems based on PC plug-in boards. - Although Microsoft Windows has become the standard operating system for PC applications, it is a non-deterministic operating system that can’t provide predictable response times in critical measurement and control applications. - Therefore, the solution is to link the PC to a system that can operate autonomously and provide rapid, predictable responses to external stimuli. Instruments - Originally, discrete electronic test instruments consisted mostly of single-channel meters, sources, and related instrumentation intended for general-purpose test applications. - Over the years, the addition of communication interfaces and advances in instrument design, manufacturing, and measurement technology have extended the range and functionality of these instruments. - New products such as scanners, multiplexers, SourceMeter® instruments, counter/timers, nanovoltmeters, micro-ohmmeters, and other specialized instrumentation have made it possible to create computer-controlled test and measurement systems that offer exceptional sensitivity and resolution. - Some systems of this type can service only one channel or just a few channels, so their cost per channel is high. - However, the addition of switch matrices and multiplexers can lower the cost per channel by allowing one set of instruments to service many channels while preserving high signal integrity. - These instruments can also be combined with computers that contain plug-in data acquisition boards. |  |
| Features of Discrete Instruments for Data Acquisition |  |  |
| 1 | Support measurement ranges and sensitivities generally beyond the limits of standard plug-in boards and external data acquisition systems. |  |
| 2 | Use standard interfaces (e.g., IEEE-488, RS-232, FireWire, USB) that support long-distance acquisition, compatibility with non-IBM-compatible computers, or use with computers without available expansion slots. |  |
| 3 | Most suitable for measurement of voltage, current, resistance, capacitance, inductance, temperature, etc. May not be effective solutions for some types of specialized sensors or signal conditioning requirements. |  |
| 4 | Generally slower than plug-in boards or external data acquisition systems. |  |
| 5 | More expensive than standard data acquisition systems on a per-channel basis. |  |
| Hybrid Data Acquisition Systems (Developed in external data acquisition system) | Hybrid Data Acquisition Systems Made Up of: - DMM-type user interface - Standard data acquisition functions - Expansion capabilities in a compact, instrument-like package Functions of Hybrid Systems - AC & DC voltage & current measurements - Temperature & frequency measurements - Event counting - Timing & triggering - Process control Keithley’s Integra Series - Includes Model 2700 and Model 2750 - Supports plugin modules - Provides multiple board slots for expansion - Expands measurement capabilities & channel capacity Key Features: - DMM front end with digital display & controls - Resolution: 18 to 22 – bit A/D or better - High accuracy, measurement range, and sensitivity (better than standard equipment) Functional Features - Built-in data & program storage - Enables stand-alone data logging - Supports process control - Uses standard interfaces (IEEE-488) - Long distance acquisition - Compatibility with non-pc computers Cost & limitation - Cost-effective (per channel) - Limited expansion capacity - Generally slower than plug-ins |  |
| Sensors and Actuators | - Embedded systems interact with the real world. - Sensors detect changes in the environment/variables. - Actuators respond by controlling or adjusting outputs. - Purpose: Monitoring or Controlling functions. Main Purpose Controlling Purpose - System produces changes in the controlling variable to reach the desired value. - Achieved via an actuator connected to the system’s output port. Monitoring Purpose - No actuator is required. - Example: ECG Machine - Monitors patient’s heart beat status. - Uses electrode sets attached to the body. - Variations are displayed visually or printed for doctors. |  |
| Sensors | - Transducer devices that convert energy from one form to another for any measurement or control purpose. - They detect and measure physical or environmental changes and convert them into electrical signals or other readable forms. - The sensory organs of electronic systems - In the IoT context, they are responsible for collecting real-world data from the environment - The data collected serves as the foundation for: - generating insights - making informed decisions - enabling smart automation |  |
| Accelerometer Sensor | - Measures acceleration forces - It counts steps for pedometer functionality |  |
| Ambient Light Sensor (ALS) | - It measures light intensity - Used in some smartwatches |  |
| Basic Working Principle |  |  |
| Analog Sensors | - Generates continuous analog signals - The output signal is proportional to the magnitude of the sensed parameter - It provides a continuous range of values, allowing for precise measurements - The analog output is in the form of voltage or current Example of Analog Sensors Temperature Sensors; Measures the ambient temperature (thermocouples, Resistance Temperature Detectors (RTDs), thermistors). Pressure Sensors; Measures the force applied by liquids or gases (strain gauges, piezoelectric sensors). Humidity Sensors; - Measures moisture content in the air - The analog output is based on capacitance and resistance changes - Commonly used in climate control systems and weather monitoring. Light Sensors (Photodetectors); Measures the intensity of light in their surroundings (photodiodes, Light-Dependent Resistors (LDRs)). Gas Sensors; Detects the presence of specific gases in the atmosphere (Indoor air quality monitoring, industrial safety). Flex Sensors; - Changes the resistance with bending or flexing - Commonly used to detect mechanical movements or deformations. Potentiometers; - Variable resistors that change their resistance by adjusting the knob or slider - Used for control and adjustment purposes. Proximity Sensors; - Detects the presence of absence of an object without physical contact. |
| Digital Sensors | - Digital sensors, on the other hand, generate discrete, digital signals representing binary data (0s and 1s). These sensors convert the analog signal into digital form using an analog-to-digital converter (ADC). - The ADC samples the analog signal at specific intervals, quantizes it into digital values, and then represents those values in binary format. - Digital sensors are more immune to noise and can be easily interfaced with microcontrollers and digital processing units. Examples of Digital Sensors Digital Temperature Sensor (Digital Thermometer); These sensors incorporate an analog temperature sensor but output the temperature value in digital form after converting the analog signal using an internal ADC. They are widely used in consumer electronics and IoT devices.. Digital Light Sensors (Digital Ambient Light Sensors); Similar to digital thermometers, digital ambient light sensors use an ADC to convert the analog light signal into digital values for displaying light levels digitally. They are used in automatic brightness control and display backlighting.. Digital Proximity Sensors; Digital proximity sensors use infrared or ultrasonic technology to measure distance to an object and provide a binary output indicating if an object is within a certain range or not. They are used in touchless switches, object detection, and robotics.. Digital Gyroscopes; Digital gyroscopes measure angular rotation and provide digital output representing the rotation rate. They are commonly used in devices like smart phones and gaming controllers for motion sensing.. Digital Accelerometer; Digital accelerometers measure acceleration and provide digital output representing the acceleration values. They are used in applications like motion tracking, orientation detection, and impact detection.. Digital Compass Sensors; These sensors measure the Earth's magnetic field and provide digital output indicating the direction or orientation relative to the Earth's magnetic north. They are used in navigation systems and digital compass applications.. Digital Magnetic Hall Effect Sensors; Digital Hall effect sensors detect changes in the magnetic field and provide digital output representing the presence or absence of a magnetic field. They are used in applications like door switches and position sensing.. Digital Touch Sensors; Digital touch sensors detect touch or proximity to a conductive object and provide digital output indicating the presence or absence of touch. They are used in touchscreens, buttons, and touch-sensitive interfaces.. |
| Difference Between Analog & Digital Sensors |  |  |
|  | ANALOG SENSORS | DIGITAL SENSORS |
| 1. SIGNAL REPRESENTATION | Analog sensors generate continuous signals that vary in voltage, current, or resistance based on the measured physical property. The output signal is proportional to the magnitude of the sensed parameter and provides a continuous range of values. | Digital sensors produce discrete signals represented in binary form (0s and 1s). The output signal is quantized and represents specific digital values, often using binary code to convey information. |
| 2. SIGNAL PROCESSING | Analog signals may require additional analog-to-digital conversion (ADC) if digital processing or data storage is needed. This conversion introduces quantization errors, potentially reducing accuracy. | Digital signals are easily processed by digital systems, such as microcontrollers, computers, and digital signal processors (DSPs). There is no need for analog-to-digital conversion, leading to higher accuracy and reduced noise susceptibility. |
| 3. NOISE IMMUNITY | Analog signals are susceptible to noise and interference during transmission and processing, leading to potential inaccuracies in measurements. | Digital signals are more immune to noise since they use discrete voltage levels, making it easier to distinguish between valid data and noise. |
| 4. INTERFACING | Interfacing analog sensors with digital systems requires ADC to convert analog signals to digital, adding complexity and potentially increasing power consumption. | Digital sensors directly interface with digital systems, eliminating the need for additional conversion circuits and simplifying the overall system design. |
| 5. PRECISION & ACCURACY | Analog sensors can offer high precision and accuracy due to the continuous range of values they provide. | Digital sensors may have slightly reduced precision due to quantization effects during analog-to-digital conversion. However, they can still achieve high accuracy in many applications. |
| 6. POWER CONSUMPTION | Analog sensors may consume less power as they do not require additional ADC circuits for data processing. | Digital sensors may be more power-efficient during communication and data processing stages, especially when interfacing with digital systems. |
| OUTPUT |  |  |
| Working of Sensors The working of a sensor depends on its type and the physical property it is designed to measure. However, most sensors operate based on a few general principles. Here's a general overview of how sensors work: |  |  |
| 1.DETECTION OF PHYSICAL PROPERTY | Sensors are designed to detect specific physical properties or environmental changes, such as temperature, pressure, light intensity, humidity, motion, proximity, or gas concentration. The sensor's construction and materials are optimized to respond to these specific properties. |  |
| 2. TRANSDUCTION | When the sensor is exposed to the physical property or environmental change, it undergoes a transduction process. Transduction refers to the conversion of the detected physical quantity into a measurable electrical signal. |  |
| 3. OUTPUT SIGNAL GENERATION | After transduction, the sensor generates an output signal in the form of an electrical voltage, current, or resistance, which represents the measured physical property. The output signal's magnitude or characteristics are directly related to the magnitude of the sensed physical property. |  |
| 4. SIGNAL CONDITIONING (Option) | In some cases, the raw output signal from the sensor may require conditioning to improve accuracy or adapt it to the specific application's requirements. Signal conditioning may involve amplification, filtering, linearization, or compensation. |  |
| 5. DATA PROCESSING & INTERPRETATION (Digital Sensors): | In the case of digital sensors, the analog output signal is converted into digital form using an analog-to-digital converter (ADC). The digital data can then be processed and interpreted by digital systems, such as microcontrollers, computers, or digital signal processors (DSPs). |  |
| 6. OUTPUT AND DISPLAY | The sensor's output can be further processed, logged, displayed on a screen, or used to control other electronic components or systems, depending on the application's requirements. |  |
| 7. FEEDBACK OR CONTROL (Closed-Loop Systems): | In some cases, sensors are used in closed-loop systems where the sensor's output is used to provide feedback to a control system. The control system can then make adjustments based on the sensor's measurements to achieve a desired outcome or maintain a specific condition. |  |
| Different sensors employ various mechanisms and technologies to achieve transduction, such as resistive changes, capacitive changes, inductive changes, piezoelectric effects, and optical sensing. Each type of sensor is optimized for specific applications based on its working principle. |  |  |
| Actuators | - Actuator is a form of transducer device (mechanical or electrical) which converts signals to corresponding physical action (motion). - Actuator acts as an output device. We can see that certain smartwatches use Ambient Light Sensor to detect the surrounding light intensity and uses an electrical/ electronic actuator circuit to adjust the screen brightness for better readability. - Actuators are devices or components that convert various forms of energy, such as electrical, hydraulic, pneumatic, or mechanical, into physical motion, force, or mechanical action. - They play a fundamental role in various systems by bringing motion to life and performing specific tasks based on input signals from control systems or sensors - Actuators are essential in creating intelligent systems that can interact with the physical world and carry out actions as needed. Types of Actuators 1.ELECTRICAL ACTUATORS; - DC Motors: Direct current (DC) motors are widely used for rotational motion. They consist of a rotor and a stator with a coil that produces a magnetic field. When current flows through the coil, a force is generated, causing the rotor to rotate. - Stepper Motors: Stepper motors move in discrete steps or increments, allowing precise control over angular position. They find applications in robotics, 3D printers, and CNC - Servo Motors: Servo motors are used in systems that require precise control over speed and position. They use a feedback loop to adjust the motor's position based on the desired setpoint. - Linear Actuators: Linear actuators provide linear motion instead of rotational motion. They are used in applications where straight such as in electric vehicles, medical devices, and home automation. - Solenoids: Solenoids are electromechanical devices that produce linear motion when an electrical current passes through a coil, generating a magnetic field. They are often used in applications like door locks, valves, and switches. - Piezoelectric actuators use piezoelectric materials that expand or contract when an electric field is applied. They are used in precise positioning systems, scanning probe microscopes, and nano positioning applications.. 2. HYDRAULIC ACTUATORS; Hydraulic actuators use pressurized hydraulic fluid to generate mechanical motion or force. They are capable of providing high force output and are commonly used in heavy machinery, construction equipment, and industrial automation.. 3. PNEUMATIC ACTUATORS; Pneumatic actuators use compressed air or gas to produce mechanical motion or force. They are often used in applications where a lightweight, fast is needed, such as in pneumatic cylinders and air-powered tools.. 4. MECHANICAL ACTUATORS; Mechanical actuators operate directly without using external energy sources such as electricity or fluids. Examples include manually operated levers, gears, and linkages.. Actuators Selection Criteria - Performance Requirements - Power Source and Energy Efficiency - Environmental Considerations - Control and Feedback Mechanisms - Safety and Reliability Applications of Actuators - Robotics - Automotive - Aerospace - Healthcare - Industrial automation Emerging trends in Actuator Technology Soft actuators and Soft Robotics; Actuators made from soft materials that mimic natural biological structures.. MEMS-Based Actuators; Micro electro-mechanical systems (MEMS) enable the integration of actuators and sensors on a microscale.. Biomimetic Actuators and Biohybrid Systems; Actuators inspired by biological systems, allowing for more natural and efficient movement.. Shape Memory Alloys in Actuators; Materials with shape memory properties that can change shape in response to external stimuli.. |
| I/O Subsystem | - The I/O subsystem of the embedded system facilitates the interaction of the embedded system with the external world. As mentioned earlier the interaction happens through the sensors and actuators connected to the input and output ports respectively of the embedded system. - The sensors may not be directly interfaced to the input ports, instead they may be interfaced through signal conditioning and translating systems like ADC, optocouplers, etc. Some Well Known I/Os Light Emitting Diode (LED); Light Emitting Diode (LED) is an important output device for visual indication in any embedded system. LED can be used as an indicator for the status of various signals or situations. Typical examples are indicating the presence of power conditions like ‘Device ON’, ‘Battery low’ or ‘Charging of battery’ for a battery-operated handheld embedded device.. 7-Segment LED Display; The 7-segment LED display is an output device for displaying alpha numeric characters. It contains 8 light-emitting diode (LED) segments arranged in a special form. Out of the 8 LED segments, 7 are used for displaying alpha numeric characters and 1 is used for representing ‘decimal point’ in decimal number display.. Optocoupler; Optocoupler is a solid-state device to isolate two parts of a circuit. Optocoupler combines an LED and a photo-transistor in a single housing (package). In electronic circuits, an optocoupler is used for suppressing interference in data communication, circuit isolation, high voltage separation, simultaneous separation and signal intensify cation, etc. Optocouplers can be used in either input circuits or in output circuits.. STEPPER MOTOR; A stepper motor is an electro-mechanical device which generates discrete displacement (motion) in response to dc electrical signals. It differs from the normal dc motor in its operation. The dc motor produces continuous rotation on applying dc voltage whereas a stepper motor produces discrete rotation in response to the dc voltage applied to it. Stepper motors are widely used in industrial embedded applications, consumer electronic products and robotics control systems. The paper feed mechanism of a printer/fax makes use of stepper motors for its functioning. RELAY; Relay is an electro-mechanical device. In embedded application, the ‘Relay’ unit acts as dynamic path selectors for signals and power. The ‘Relay’ unit contains a relay coil made up of insulated wire on a metal core and a metal armature with one or more contacts. ‘Relay’ works on electromagnetic principle. When a voltage is applied to the relay coil, current flows through the coil, which in turn generates a magnetic field. The magnetic field attracts the armature core and moves the contact point. The movement of the contact point changes the power/signal flow path. Relays are available in different configurations.. PIEZO BUZZER; Piezo buzzer is a piezoelectric device for generating audio indications in embedded application. A piezoelectric buzzer contains a piezoelectric diaphragm which produces audible sound in response to the voltage applied to it. Piezoelectric buzzers are available in two types: ‘Self-driving’ and ‘External driving’.. PUSH BUTTON SWITCH; It is an input device. Push button switch comes in two configurations, namely ‘Push to Make’ and ‘Push to Break’.. KEYBOARD; Keyboard is an input device for user interfacing. If the number of keys required is very limited, push button switches can be used and they can be directly interfaced to the port pins for reading. However, there may be situations demanding a large number of keys for user input (e.g. PDA device with alpha-numeric keypad for user data entry).. PROGRAMMABLE PERIPHERAL INTERFACE (PPI); Programmable Peripheral Interface (PPI) devices are used for extending the I/O capabilities of processors/controllers. Most of the processors/controllers provide a very limited number of I/O and data ports, and at times it may require a greater number of I/O ports than the one supported by the controller/processor. A programmable peripheral interface device expands the I/O capabilities of the processor/controller.. |

| Category | Items / Details |
| --- | --- |
| USART Operations | - Enable USART - Disable USART transmitter |
| Synchronous Protocols | - I2C Protocol - SPI Protocol - CAN Protocol - USB - Micro Wire |
| Asynchronous Protocols | - 1-Wire - RS232 - RS422 - RS485 (Line Driver) |
| Synchronous vs Asynchronous (Reminder) | Synchronous: Uses clock signal for timing. Asynchronous: No shared clock; uses start/stop bits. |
| Types of Serial Ports | - Synchronous Serial Input - Synchronous Serial Output - Asynchronous Serial UART Input - Asynchronous Serial UART Output |
| Duplex Communication | - Full Duplex: Can send and receive data simultaneously |

| Category | Items / Details |
| --- | --- |
| Analog Sensors | - Temperature Sensors - Pressure Sensors - Humidity Sensors - Light Sensors - Gas Sensors - Flex Sensors - Potentiometer - Proximity Sensors |
| Digital Sensors | - Digital Temperature Sensors - Digital Light Sensors - Digital Proximity Sensors - Digital Gyroscopes - Digital Accelerometer - Digital Compass Sensors - Digital Magnetic Hall Effect Sensor - Digital Touch Sensor |
| Difference Between Analog & Digital Sensors | - Signal Representation - Signal Processing - Noise Immunity - Interference - Precision & Accuracy - Power Consumption |
| Working of Sensors | - Detection of Physical Property - Transduction - Output Signal Generation - Signal Conditioning - Data Processing & Interpretation - Output & Display - Feedback or Control |
| Types of Actuators | - Electrical Actuators - Hydraulic Actuators - Pneumatic Actuators - Mechanical Actuators |
| Electrical Actuators | - DC Motors - Stepper Motors - Servo Motors - Linear Motors - Solenoids - Piezoelectric Actuators |
| Mechanical Actuators | - Levers - Gears - Linkages |
| Applications of Actuators | - Robotics - Automotive - Aerospace -Healthcare - Industrial Automation |
| Interrupts (General Topics) | - Watchdog - Analog and Digital Sensors - RTC Battery Type: CR2032 - Vectored- Timer Mode 3 |
| Classification of Interrupts | 1. External and Internal Interrupts 2. Maskable and Non-Maskable Interrupts 3. Vectored and Non-Vectored Interrupts |