Southern Luzon State University

COLLEGE OF ENGINEERING

Computer Engineering Department

Lucban, Quezon

CPE24L – Embedded Systems Assignment

Submitted by:

ESQUILLO, Jay Anthony L.

MAÑAGO, John Oshrick S.

PADUADA, Dave Jhared G.

Engr. Jose Yulo Manzanero

INSTRUCTOR

Fritzing Schematic

App Screenshot

App Code

package com.example.rccar

import android.annotation.SuppressLint
import android.bluetooth.BluetoothAdapter
import android.bluetooth.BluetoothDevice
import android.bluetooth.BluetoothManager
import android.bluetooth.BluetoothSocket
import android.content.Context
import android.os.Handler
import android.os.Looper
import android.util.Log
import java.io.IOException
import java.io.OutputStream
import java.util.UUID

object BluetoothService {
    private const val TAG = "BluetoothService"
    private val MY_UUID: UUID = UUID.fromString("00001101-0000-1000-8000-00805F9B34FB") // Standard SerialPortService ID

    // Command constants (Arduino expects single character commands)
    const val CMD_LED_ON = "1"
    const val CMD_LED_OFF = "2"
    const val CMD_BUZZER_ON = "3"
    const val CMD_BUZZER_OFF = "4"
    const val CMD_LCD_ON = "5"
    const val CMD_LCD_OFF = "6"
    const val CMD_SERVO_ON = "7"
    const val CMD_SERVO_OFF = "8"

    var bluetoothAdapter: BluetoothAdapter? = null
    private var bluetoothSocket: BluetoothSocket? = null
    private var outputStream: OutputStream? = null

    var isConnected = false
        private set

    fun init(context: Context) {
        val bluetoothManager = context.getSystemService(Context.BLUETOOTH_SERVICE) as BluetoothManager
        bluetoothAdapter = bluetoothManager.adapter
    }

    @SuppressLint("MissingPermission")
    fun connect(device: BluetoothDevice, onResult: (Boolean) -> Unit) {
        bluetoothAdapter?.cancelDiscovery()
        Thread {
            try {
                bluetoothSocket = device.createRfcommSocketToServiceRecord(MY_UUID)
                bluetoothSocket?.connect()
                outputStream = bluetoothSocket?.outputStream
                isConnected = true
                Handler(Looper.getMainLooper()).post { onResult(true) }
            } catch (e: IOException) {
                Log.e(TAG, "Connection failed", e)
                closeConnection()
                Handler(Looper.getMainLooper()).post { onResult(false) }
            }
        }.start()
    }

    fun sendCommand(command: String) {
        if (!isConnected || outputStream == null) {
            Log.w(TAG, "sendCommand ignored; not connected. cmd=$command")
            return
        }
        try {
            // Send raw command; Arduino handles trimming/newlines if present
            val payload = command.toByteArray()
            outputStream?.write(payload)
            outputStream?.flush()
            Log.d(TAG, "Command sent: $command")
        } catch (e: IOException) {
            Log.e(TAG, "Error sending command", e)
            closeConnection()
        }
    }

    fun closeConnection() {
        try {
            outputStream?.close()
            bluetoothSocket?.close()
            isConnected = false
        } catch (e: IOException) {
            Log.e(TAG, "Error closing connection", e)
        }
    }
}

package com.example.rccar

import android.content.pm.ActivityInfo
import android.os.Bundle
import android.view.LayoutInflater
import android.view.View
import android.view.ViewGroup
import androidx.fragment.app.Fragment
import com.example.rccar.databinding.FragmentActivityBinding

class ActivityFragment : Fragment() {
    private var _binding: FragmentActivityBinding? = null
    private val binding get() = _binding!!

    override fun onCreateView(
        inflater: LayoutInflater, container: ViewGroup?,
        savedInstanceState: Bundle?
    ): View {
        _binding = FragmentActivityBinding.inflate(inflater, container, false)
        return binding.root
    }

    override fun onViewCreated(view: View, savedInstanceState: Bundle?) {
        super.onViewCreated(view, savedInstanceState)

        binding.toggleLed.setOnCheckedChangeListener { _, isChecked ->
            BluetoothService.sendCommand(if (isChecked) BluetoothService.CMD_LED_ON else BluetoothService.CMD_LED_OFF)
        }

        binding.toggleBuzzer.setOnCheckedChangeListener { _, isChecked ->
            BluetoothService.sendCommand(if (isChecked) BluetoothService.CMD_BUZZER_ON else BluetoothService.CMD_BUZZER_OFF)
        }

        binding.toggleServo.setOnCheckedChangeListener { _, isChecked ->
            BluetoothService.sendCommand(if (isChecked) BluetoothService.CMD_SERVO_ON else BluetoothService.CMD_SERVO_OFF)
        }

        binding.toggleLcd.setOnCheckedChangeListener { _, isChecked ->
            BluetoothService.sendCommand(if (isChecked) BluetoothService.CMD_LCD_ON else BluetoothService.CMD_LCD_OFF)
        }
    }

    override fun onDestroyView() {
        super.onDestroyView()
        requireActivity().requestedOrientation = ActivityInfo.SCREEN_ORIENTATION_UNSPECIFIED
        _binding = null
    }
}

Arduino Code

#include <Servo.h>

#include <LiquidCrystal_I2C.h>

#include <Wire.h>

// --- PIN DEFINITIONS ---

const int LED_RELAY_PIN      = 8;   // Relay CH1 - LED

const int BUZZER_RELAY_PIN   = 9;   // Relay CH2 - Buzzer

const int LCD_RELAY_PIN      = 10;  // Relay CH3 - LCD Power

const int SERVO_POWER_PIN    = 12;  // Relay CH4 - Servo Power

const int SERVO_SIGNAL_PIN   = 11;  // Servo Signal Pin

// --- GLOBAL OBJECTS ---

Servo myServo;

LiquidCrystal_I2C lcd(0x27, 16, 2);

// --- VARIABLES ---

char incomingByte = 0;

void setup() {

// Initialize serial ports

Serial.begin(9600);

Serial1.begin(9600);

// Initialize relay pins

pinMode(LED_RELAY_PIN, OUTPUT);

pinMode(BUZZER_RELAY_PIN, OUTPUT);

pinMode(LCD_RELAY_PIN, OUTPUT);

pinMode(SERVO_POWER_PIN, OUTPUT);

// Attach servo

myServo.attach(SERVO_SIGNAL_PIN);

myServo.write(0); // Initial servo angle

Serial.println("System Initialized. All Relays OFF.");

delay(1000);

// All relays OFF (HIGH = OFF)

digitalWrite(LED_RELAY_PIN, HIGH);

digitalWrite(BUZZER_RELAY_PIN, HIGH);

digitalWrite(LCD_RELAY_PIN, HIGH);

digitalWrite(SERVO_POWER_PIN, HIGH);

}

void loop() {

if (Serial1.available() > 0) {

incomingByte = Serial1.read();

Serial.print("Received Command: ");

Serial.println(incomingByte);

switch (incomingByte) {

// --- LED Control (Relay 1) ---

case '1':

digitalWrite(LED_RELAY_PIN, LOW);

Serial.println("-> LED ON");

break;

case '2':

digitalWrite(LED_RELAY_PIN, HIGH);

Serial.println("-> LED OFF");

break;

// --- Buzzer Control (Relay 2) ---

case '3':

digitalWrite(BUZZER_RELAY_PIN, LOW);

Serial.println("-> BUZZER ON");

break;

case '4':

digitalWrite(BUZZER_RELAY_PIN, HIGH);

Serial.println("-> BUZZER OFF");

break;

// --- LCD Power Control (Relay 3) ---

case '5':

digitalWrite(LCD_RELAY_PIN, LOW);

Serial.println("-> LCD POWER ON (Relay K3 Activated)");

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Merry Christmas");

lcd.setCursor(0, 1);

lcd.print("System Active");

break;

case '6':

digitalWrite(LCD_RELAY_PIN, HIGH);

Serial.println("-> LCD POWER OFF (Relay K3 Deactivated)");

break;

// --- Servo Power + Movement (Relay 4) ---

case '7':  // Servo ON

digitalWrite(SERVO_POWER_PIN, LOW);

myServo.write(90);

delay(1000);

Serial.println("-> Servo Power ON (K4 ON) & 90 deg command sent.");

break;

case '8':  // Servo OFF

Serial.println("-> Servo Power OFF (K4 OFF)");

myServo.write(0);

delay(1000);

digitalWrite(SERVO_POWER_PIN, HIGH);

break;

// --- Unknown Command ---

default:

Serial.println("-> Unknown command.");

break;

}

}

}